Frequency Masking

A frequency mask is a filter that selectively allows or blocks specific frequency ranges in a signal — essentially a "template" that defines which frequencies to keep (pass) or remove (suppress).

Think of it like cutting holes in a stencil — only frequencies aligned with the "holes" get through.

Types:

| Mask Type | Keeps | Removes |
| --- | --- | --- |
| Low-pass | Low frequencies | High frequencies |
| High-pass | High frequencies | Low frequencies |
| Band-pass | A specific range | Everything outside |
| Band-stop (notch) | Everything outside | A specific range |
| Custom | Arbitrary bins | Whatever you define |

How it works step by step:

- Load the .wav into a NumPy array

- FFT converts the signal from time → frequency domain (you can now "see" individual frequencies)

- Mask zeroes out unwanted frequency bins

- IFFT converts back to time domain with those frequencies removed

- Save/analyse the cleaned signal