Frequency Masking

A frequency mask is a filter that selectively allows or blocks specific frequency ranges in a signal — essentially a "template" that defines which frequencies to keep (pass) or remove (suppress).

Think of it like cutting holes in a stencil — only frequencies aligned with the "holes" get through.

Types:

| Mask Type | Keeps | Removes |
| --- | --- | --- |
| Low-pass | Low frequencies | High frequencies |
| High-pass | High frequencies | Low frequencies |
| Band-pass | A specific range | Everything outside |
| Band-stop (notch) | Everything outside | A specific range |
| Custom | Arbitrary bins | Whatever you define |

Implementing in Python with a .wav file

import numpy as np

from scipy.io import wavfile

from scipy.fft import fft, ifft

import matplotlib.pyplot as plt

# --- Load the .wav file ---

sample_rate, data = wavfile.read("venus_audio.wav")

# Convert stereo to mono if needed

if data.ndim > 1:

data = data.mean(axis=1)

# --- FFT: Time domain → Frequency domain ---

fft_data = fft(data)

frequencies = np.fft.fftfreq(len(fft_data), d=1/sample_rate)

# --- Build a frequency mask ---

# Example: Band-pass — keep only 100Hz to 800Hz

mask = np.zeros(len(frequencies), dtype=bool)

mask[(np.abs(frequencies) >= 100) & (np.abs(frequencies) <= 800)] = True

# --- Apply the mask ---

fft_masked = fft_data * mask  # Zero out everything outside the band

# --- IFFT: Back to time domain ---

filtered_signal = np.real(ifft(fft_masked))

# --- Save the result ---

filtered_signal = np.int16(filtered_signal / np.max(np.abs(filtered_signal)) * 32767)

wavfile.write("filtered_output.wav", sample_rate, filtered_signal)

# --- Visualise ---

plt.figure(figsize=(12, 4))

plt.plot(np.abs(frequencies[:len(frequencies)//2]),

np.abs(fft_data[:len(fft_data)//2]), alpha=0.5, label="Original")

plt.plot(np.abs(frequencies[:len(frequencies)//2]),

np.abs(fft_masked[:len(fft_masked)//2]), label="Masked")

plt.xlabel("Frequency (Hz)")

plt.ylabel("Amplitude")

plt.legend()

plt.title("Frequency Mask Applied")

plt.show()

How it works step by step:

- Load the .wav into a NumPy array

- FFT converts the signal from time → frequency domain (you can now "see" individual frequencies)

- Mask zeroes out unwanted frequency bins

- IFFT converts back to time domain with those frequencies removed

- Save/analyse the cleaned signal

Custom mask example (e.g. for Venus wind isolation):

# Suppress engine/instrument noise below 20Hz and above 1000Hz

mask = (np.abs(frequencies) >= 20) & (np.abs(frequencies) <= 1000)

fft_masked = fft_data * mask

The key insight is that multiplying by 0 in frequency space = completely removing that frequency from the signal.